Patch to CP/M-68K (tm) C Run-time Library

The initial shipments of CP/M-68K had a problem in writing Ascii

files using the C run-time library.  The patch described below

fixes the problem.

A>AR68 X CLIB WRITE.O	(extract module WRITE.O from library CLIB)

A>DDT

-RWRITE.O		(read file WRITE.O into memory)

start xxxxxxxx  end yyyyyyyy

-Hxxxxxxxx,164		(xxxxxxxx is the start in memory of the file)

zzzzzzzz wwwwwwww	(zzzzzzzz is the sum of 164H and xxxxxxxx)

-Dzzzzzzzz		(display memory at zzzzzzzz)

zzzzzzzz: 67 18 33 FC ...  	(memory contents)

...

...

-Szzzzzzzz

zzzzzzzz 67 60		(change byte from 67 to 60)

uuuuuuuu 18 .		(display next byte -- don't change)

-WWRITE.O		(write out changed file)

-^C			(Control-C to exit DDT)

A>AR68 RV CLIB WRITE.O	(Replace module WRITE.O in library CLIB)

...			(Archiver types confirmation messages)

A>ERA WRITE.O

The effect of the above sequence is to replace one byte in the run-time

library.  The byte changed is at offset 164H in object module WRITE.O,

from 67H to 60H.