CP/M-68K Cross Software for VMS

-------------------------------

Revision 01.04

--------------

August 28, 1983

---------------

Digital Research, Inc

P.O. Box 579

Pacific Grove, CA  93950

(408) 649-3896

TWX 910 360 5001

************************

* Company Confidential *

************************

Note:  This document contains Digital Research proprietary infor-

mation,  and  must  not  be copied or distributed for any purpose

whatsoever, without prior written permission of Digital Research,

Inc.

Address Comments to:  Steve Williams, x6082

CP/M-68K Cross Software for VMS                            Page 2

Introduction

1.0  Introduction

This  document  describes  the  installation  and  execution

procedures  for  the  Digital  Research  68000  cross development

software for VAX/VMS.  This software  includes  the  C  compiler,

assembler,  linker,  archiver, symbol table print utility, object

module size utility,  and  S-Record  conversion  utility.   These

tools execute in much the same fashion as they do under CP/M-68K,

as documented in the CP/M-68K Programmer's Guide.

2.0  Installation Procedures

The tools are distributed on a 9-track FLX format  mag  tape

(either  800 or 1600 bpi is available).  To bring up the software

on VMS, perform the following steps:

1. Login to a valid account.

2. Create directories for four ersatz devices:  "bin:"  (.exe

files),  "com:"  (command  files), "lib:" (library files),

and "cpm:" (CP/M-68K files).

3. Enter VMS  "ASSIGN"  commands  for  these  directories  in

LOGIN.COM, e.g:

$ assign drb0:[c.bin] bin:

$ assign drb0:[c.com] com:

$ assign drb0:[c.lib] lib:

$ assign drb0:[c.cpm] cpm:

Note that the full path name must be specified.

4. Perform  the  above  assignments  manually,  re-login   or

re-execute LOGIN.COM to make the device names valid.

5. Read the tape as follows:

$ allocate mt:

$ mount mt:/foreign

$ set def lib:

$ mcr flx /rs=mt:[1,1]*.*/do/im

$ mcr flx /rs=mt:[1,2]*.*/do

$ set def bin:

$ mcr flx /rs=mt:[1,3]*.*/do

$ set def cpm:

$ mcr flx /rs=mt:[1,4]*.*/do/im

$ mcr flx /rs=mt:[1,5]*.*/do

$ set def com:

$ mcr flx /rs=mt:[1,6]*.*/do

$ @install

$ dismount mt:

$ deallocate mt:

CP/M-68K Cross Software for VMS                            Page 3

Installation Procedures

You may need to replace the "mt:" device name above with a

different device name, depending on the type of tape drive

you have.  Consult your system administrator for details.

6. Add the statements in the file "xlogin.com"  to  LOGIN.COM

so  that  the  proper command definitions are performed at

login time.

Once the above steps have been performed, the software  is  ready

to use on VMS.

3.0  Operating Instructions

The following sections describe how  to  run  the  installed

cross software on VMS.

3.1  C compilation

A C compilation is performed  using  the  'c68'  command  as

follows:

$ c68 file

The ".c" extension is implied and must  not  be  specified.   The

'c68'  command  is  extremely  similar  to the 'C' SUBMIT file on

CP/M-68K.  Default floating point format is  Motorola  FFP  using

the "c68" command.  The "ce68" command causes the compiler to use

IEEE format floating point numbers.

3.2  Assembly

The assembler is invoked in exactly the same  manner  as  on

CP/M-68K.  The output (listing and errors) can be redirected to a

file or device using the ">filename" construct.  A sample command

is:

$ as68 -l -u -p file.s >file.lis

This  command  assembles  "file.s"  and  places  the  object   in

"file.o".   The  listing  is output to "file.lis".  All assembler

options valid under CP/M-68K may be used on VMS as well.

3.3  Linking

The LO68 program is used with the same options  and  command

line syntax as on CP/M-68K.  For example,

$ lo68 -r -o file.rel file1.o file2.o

links "file1.o" with "file2.o", creating "file.rel",  a  relocat-

able format file.

CP/M-68K Cross Software for VMS                            Page 4

Operating Instructions

C programs are linked using the  "clink"  command,  as  with

CP/M-68K:

$ clink file1

This links "file1.o" with  the  C  runtime  software  to  produce

"file1.rel",  a relocatable format 68000 load module.  Separately

compiled modules may be linked in the following way:

$ clink file1 file2 file3 file4 ...

All of the object files are linked  to  produce  "file1.rel".   A

limit of 9 such files is imposed by the command file.

The "clink" command procedure assumes that the program being

linked  does not use floating point.  To link a program using FFP

floating point, use "clinkf" instead of "clink".   IEEE  floating

point  programs  (those  compiled  with  the  "ce68" command) are

linked using "clinke".

3.4  Archive Files

The CP/M-68K archive program (AR68) can be  used  under  VMS

exactly  in  the same fashion as under CP/M-68K.  For example, to

create a library file:

$ ar68 r lib a.o b.o c.o

This command creates a library called "lib" containing the  three

files "a.o", "b.o", and "c.o".

4.0  Building CP/M-68K

Using the VAX cross tools to build CP/M-68K  is  similar  to

building  the  operating  system on CP/M.  You link the BIOS with

CPMLIB to produce a relocatable image file  (CPM.REL).   Use  the

RELOC  utility  to create the absolute load image (CPM.SYS).  The

CP/M-68K SUBMIT  files  "LCPM.SUB"  and  "RELCPM.SUB"  have  been

adapted to VMS procedure files to make this process easier.

For example, to create the EXORmacs distribution system, use

the following sequence of commands:

$ set def cpm:

$ @normbios      !Create "bios.o"

$ @lcpm  !Create "cpm.rel"

$ @relcpm 15000 !Create CPM.SYS at 15000 hex.

The boot loader for CP/M-68K is generated in a  similar  fashion,

as documented in the CP/M-68K System Guide.

CP/M-68K Cross Software for VMS                            Page 5

File Transfer Techniques

5.0  File Transfer Techniques

5.1  S-records

The normal method of transferring files to a 68000 target is

via  Motorola  S-records.   The  SENDC68  program converts a load

module file from binary format to S-record format.   The  command

syntax for this program is:

SENDC68 [-] file.68k >file.sr

Where "file.68k" is an absolute format CP/M-68K load module,  and

"file.sr"  is  the  created S-record file.  This form of the load

module may be created using the linker "-t" switch or  the  reloc

utility.   The  RELCPM  command  used  in the CP/M-68K generation

procedure generates an absolute file.  The "-"  option  specifies

that S-records are not to be created for the ".bss" segment.  The

default is to create S-records that zero  all  locations  in  the

".bss".

The resulting S-record file may be downloaded  via  the  VMS

"TYPE" command into a target system with a PROM debugger, such as

the MACSbug program on the EXORmacs.

5.2  Other Transfer Techniques

Other techniques such as binary download utilities  or  CP/M

file  transfer  utilities  may  be used to transfer CP/M-68K load

modules to the 68000 target.  When such a technique is used,  the

bytes  in the file must be swapped, and the words of all longword

quantities in the load module header must  be  swapped.   Library

modules  may  not  be  transferred  using  this technique without

extensive rearrangement of the  headers.   The  symbol  table  in

object  and  load  module  files  will have the symbol names byte

swapped in this type of transfer.

6.0  Trademarks

VAX, VMS,  and  FLX  are  trademarks  of  Digital  Equipment

Corporation.   EXORmacs,  and MACSbug are trademarks of Motorola,

Inc.  CP/M-68K is a trademark of Digital Research, Inc.