UUCPbb MANUAL                                    TABLE OF CONTENTS

1          Introduction                                1

1.1           Hardware Requirements                    2

1.1.1            Serial Ports                          2

1.1.2            Disk Drives                           2

1.1.3            Modems                                3

1.1.4            Other Software                        3

2          This Manual                                 4

3          Setting Up                                  5

3.1           Basics                                   5

3.1.1            Programs Needed                       5

3.1.2            Required Commands                     5

3.1.3            Installation Procedure                6

3.2           Parameters File                          8

3.2.1            Node Name                             8

3.2.2            Sitename                              9

3.2.3            Other Parameters                      9

3.3           Dialers File                             9

3.4           Devices File                             9

3.5           bin.list File                           10

3.6           Systems File                            10

3.7           Miscellaneous Files                     10

3.7.1            Environment Variables                10

3.7.2            Other Files                          11

3.7.3            Password File                        11

3.8           Installing Mail                         12

3.8.1            New Installations                    12

3.8.2            Upgrading Rick's Mail                12

3.8.3            Upgrading Palm                       12

3.9           CRON/MCRON                              12

3.10          Chat Scripts                            13

3.11          Preventing Lockups                      14

4          Testing                                    15

4.1           Finding a Feed                          15

4.2           Running as Master                       16

4.3           Running as Slave                        18

5          Setting Up News                            19

5.1           Turning On Your News Feed               20

6          UUCP?  What is it?  How Does It Work?      20

6.1           What UUCP is NOT                        20

6.2           What UUCP is                            21

6.3           How Do You Get There From Here?         23

7          UUCP Command                               23

8          Fileserver                                 24

9          Final Word                                 24

1    Introduction                                        OS-9 UUCP

1 - Introduction

UUCP (UNIX-to-UNIX Copy) has been used for many years on UNIX

systems.  UUCP has developed a worldwide network of computers

which pass e-mail and Usenet news.  For a long while, OS-9

users were left out.  This all changed when Mark Griffith and

Rick Adams separately wrote their own original OS-9/6809 UUCP

programs for the CoCo allowing us to join the fun.

Rick's code (and UCPbb) is patterned after HoneyDanBer (HDB)

UUCP, and was written solely by him.  It is the code that he

uses at my UUCP site, "ccentral", running on a Color Computer 3

with 40 meg hard disk.  "Ccentral" was the first Coco3

UUCP/Usenet node to be officially registered on Usenet.

Not being one to leave well enough alone :-), I worked on

improving the software.  UUCPbb is the result.  This suite of

programs is based, mostly, on Rick's work with some parts and

ideas borrowed from Mark's original programs.  My idea was to

make the programs more flexible, easier to install, maintain

and add more features.  I hope others will find the changes

worth the wait.

I want to thank both Rick and Mark for first efforts.  I

learned a lot from studying and trying to improved on their

work.  Both Rick and Mark gave me permission to modify their

code and release it to the OS-9 Community.

This UUCP enables you to call into another remote site running

UUCP and exchange email.  This will network your Color Computer

into a worldwide network of hundreds of thousands of UUCP

sites, and give you the ability to exchange email and Usenet

news with anyone on those sites.

UUCPbb would not be where it is without the help of my Beat'n

Bash (tm) team.  These fellow help quite a lot with comments

and criticisms to make everything even better.  Boisy Pitre

<boisy@os9er.waukee.ia.us> did the port to OS-9/68K.  Boisy and

Brad Spencer <brad@anduin.eldar.org> both worked hard on

getting UUCICO to properly talk to a number of other versions

of UUCP as well as squash other bugs.  I hope we nailed them

all.  Chuck Owens <chuck@czos9.trystero.com>, Jeff Shepler

<sysop@miliways.aldhfn.org> and Tom Kocourek

<tomk@westgac3.dragon.com> also helped in finding and squashing

bugs along with their suggestions for changes.  Jeff ported his

mailer, Palm, to work with UUCPbb. Now users have a choice of

mailers.  Thanks Jeff!

Thanks for all the help gang!  Now let's make this thing even

better.  :-)

UUCPbb MANUAL                                                    2

1.1 - Hardware Requirements

The UUCPbb package will compile and run on OS-9/6809 and

OS-9/68K systems.  The minimum requirements are:

o  Any 6809 or 6309 microprocessor machine running

OS-9/6809 Level 2.  Level 1 will NOT work.

o  Any 68xxx microprocessor running OS-9/68K.

o  Any system running OS-9000 although at this time, the

code has not been tested under OS-9000. It probably

requires minor tweaking in order to compile.  This is

in the works.

o  At least 512K of RAM. It should be possible to run

UUCPbb on a 128K CoCo, but this has never been tested.

o  A hard drive or large floppy disk (720K or greater)

with enough room to install the system.

o  A Hayes compatible modem

1.1.1 - Serial Port

A hardware serial port is a must for UUCP. The "bit banger"

port simply will not work.  UUCP does not necessarily require a

modem in order to work.  Two machines running UUCP can have

direct serial connection between them.  This quite common in

the UUCP world.  It can also be done with UUCPbb.  Setting up

such a connection will be covered later.

1.1.2 - Disk Drives

A hard drive is highly recommended.  The size of the drive

depends largely on how much, if any, Usenet news you plan on

receiving.  Netnews can take up a large chunk of disk space.

(Approximately 30 MB of news is passed around Usenet every

day.) If you are only planning on receiving little or no news,

UUCP can be comfortably run on a 10 or 20 meg hard drive.  This

is, of course, assuming the hard drive is not almost full.

UUCP can also be run on a floppy-based system.  This is not

recommended as UUCP is highly disk intensive.  Floppies are

rather slow on a CoCo 3. A no-halt floppy controller is almost

a requirement on a CoCo. I say "almost" because UUCP can be

made to run using the older halting (Tandy, etc) controllers.

The setup is not pretty, but it can be made to work.  I know.

I had to use it during a hard disk crash.  If you are really

interested in the details, I'll be happy to pass them along.

:-)

A floppy system needs at least two 720K drives.  Because of the

limited storage, such a system is limited to e-mail.  News is

very unlikely to work.

3    Modems                                              OS-9 UUCP

1.1.3 - Modems

If you will be communicating with another UUCP system by phone,

you will need a modem.  Most of the modems sold today should

work provided they have the following:

o  Support the Hayes command set.

o  The ability to recall the default modem configuration

when the ATZ0 command is issued or the DTR line between

your computer and the modem is dropped.

If your modem lacks these features, UUCP may not work

properly.  If you plan on allowing systems to call in, your

modem should support auto-answering.  This is not an absolute

requirement.  Many UUCP sites only call out for security

reasons.  Lack of an auto-answer feature will prevent you from

running UUCP if you are the one doing the calling.

1.1.4 - Other software

You must also have the following software:

o  cron, mcron or some other background task scheduler.

MCRON is included with UUCPbb

o  TSMon or similar program to monitor the port for

incoming calls.  This is only need if you will be

accepting incoming UUCP calls.  If you will only be

calling out, TSMON is not necessary.

o  (CoCo only) Shell+ v2.1 or later.  UUCPbb makes heavy

use of Shell+'s features.  The stock Microware shell on

the CoCo should work.  It has not been tested however.

In addition, UUCPbb as it comes uses the following default

directories:

/DD/SYS/UUCP - The UUCP system files directory

/DD/LOG      - The log files directory

If /dd is not your primary drive, you will need to edit the

uucp.h file before compiling.

The mail, spool and news directories are user configurable.

The procedure for doing this will be described later.

UUCPbb MANUAL                                                    4

2 - This manual

This manual is not intended to teach your all about the inner

works of UUCP, networks or electronic mail; although a basic

description of UUCP will be given toward the end.  It assumes

you have some basic understanding of how to use OS-9; things

such as creating/editing files, creating directories and

setting the permissions of file and directories.

Various utilities are included in the package to make

administering UUCP relatively easy.  Running a UUCP site can be

complex, though it need not be.  Sometimes it can be difficult

getting a UUCP started.  Once started UUCP usually runs without

problem.

I will be happy to help in any way I can.  If you questions

about UUCPbb in general or on CoCo specifically, you can

contact me.  If you have questions about running UUCPbb on

under OSK (OS-9/68K), you can contact Boisy Pitre.  Right now,

I don't have an OSK system :-( so I cannot answer questions

related to OSK specific problems.  Boisy's address is:

boisy@os9er.waukee.ia.us

If you have questions or problems related related to UUCICO not

wanting to properly talk to other UUCICOs, follow the

instructions in UUCICO.MAN on how to report the problems.  You

can send these to me or Brad Spencer at brad@anduin.eldar.org.

UUCPbb is not public domain.  It is copylefted software

released under the GNU General Public License. A copy of the

GNU GPL is included in the package.  See the file COPYING for

your rights under the GNU license.  You cannot sell UUCPbb. You

are free to modify the code and give copies of this away.  Both

Rick and I would like to see UUCPbb become an OS-9 Community

project.  The details of this will be described later.

Share and enjoy!

Bob Billson (1994 April 2)

internet: bob@kc2wz.bubble.org

UUCP:     uunet!kc2wz!bob

5    Setting Up                                          OS-9 UUCP

3 - Setting Up

3.1 - Basics

3.1.1 - Programs Needed

uucico      call a UUCP neighbor and send/receive files

uucp        queue a job to transfer a file to or from a UUCP neighbor

mailx       read pending mail, or queue up mail to be sent

Palm        Jeff Shepler's mailer used in place of mailx (optional)

readnews    read Usenet news articles

postnews    queue a job to post a Usenet news article

expire      delete outdated Usenet news articles

uuxqt       perform incoming work queued up to be done

rmail       deliver mail

rnews       deliver news

uuclean     clean uucp spool directories

uulog       view the UUCP or fileserv log files

fileserv    mini-listserver

adduser     create user accounts

login       logon to the system (CoCo optional)

cnvrtmail   convert Rick's older style mail to new format (CoCo)

subscribe   get newsgroups the system receives

unsubscribe stop reading particular newsgroups

uuencode    encode binary files for mailing

uudecode    restore encoded binary files received

chown       change ownership of file/directory (CoCo)

3.1.2 - Required Commands

I have tried to make it easy to set up UUCP on both the CoCo

and OSK machines.  It will require you to create and edit some

files.  The following step-by-step instructions will guide you

through the installation.  The procedure is identical on

OS-9/6809, OS-9/68K and OS-9000 systems.  ==NOTE== you must be

the superuser (user ID 0) when through the whole procedure.

Where there are differences, the following notation will be

used:

COCO  Indicates instructions apply to the CoCo only.

Either new installation or upgrades from Rick's

original UUCP.

RICK  Indicates instructions only apply to a CoCo where

Rick's original UUCP program is already installed.

OSK   Indicates instructions which apply only to OSK

systems.

You need the following standard OS-9 commands:

RENAME, COPY, LOAD, LIST, UNLINK, and MAKDIR

You also need the C compiler, MAKE utility and your

favorite text editor.

UUCPbb MANUAL                                                    6

COCO -Highly recommended is using Carl Kreider's

replacement of the C compiler's cc1, CC.

Version 2.5.0 is available on

chestnut.cs.wisc.edu.  The file name is

cc250.lzh.  REQUIRED, however, is Carl's

replacement for the original Microware clib.l

library.  The latest version is on

chestnut.cs.wisc.edu as the file clib1990.lzh.

The original Microware clib.l cannot be used to

compile UUCPbb.

OSK  -Either Microware's Ultra C compiler or C

compiler v3.2 can used.

All the necessary commands must be in your current

execution directory.

3.1.3 - Installation Procedure

Step 1   If your current data directory, does not contain the C

source code already, change to the proper directory.

The docs files for UUCPbb are in the directory DOCS.

DOCS has three subdirectories: MAN, DOC and SAMPLE.

MAN and DOC contain files read to be printed.  SAMPLE

contains examples of the various files which will

later be copied to another directory.  It would be a

good idea to print out the files in the MAN and DOCS

directories.  They will be referred to shortly.

Step 2   Using your text editor, the following instruction

applies to the file: uucp.h.

There are certain areas of this file you should not

change.  These are clearly marked.  Read through

uucp.h to see if you need to make any changes for your

system.  The defaults should work for most systems.

If you do not have a /DD descriptor in your bootfile

or /DD is not your main drive, you may need to change

the pathnames in parts of uucp.h.  For example, there

may be no /DD/SYS on your /DD drive.  You may need to

make it /H0/SYS instead.

DO NOT change thing anything in parts marked off

limits.  Evil things may befall you otherwise.

OSK   Go to Step 4.

Step 3   COCO  -Using your text editor, edit the file:

makefile.coco.  If you are not using Carl Kreider's

CC, edit the 'CC = cc' line to the appropriate C

compiler executive.  For example, if you are using the

stock Microware C comiler, this line would be changed

7    Installation Procedure                              OS-9 UUCP

to 'CC = cc1'.

You may need to make other changes to the makefile.

The comments in makefile.coco will help you.  The '#'

is a comment.  Anything to the right of the '#' is

ignored by MAKE. To uncomment something simply delete

the '#' just to the left of the desired item.

Step 4   You are ready to compile.  The makefile you are going

to use depends on your system.  For the CoCo, you

use: makefile.coco.  If you compiling using Ultra C,

use: makefile.ucc.  If you are using the OSK C

compiler v3.2, you use: makefile.c32.  So you run:

make -f <your_makefile>

COCO  -Compile time on the CoCo is approximately one

hour.

Step 5   You are now ready to create the needed directories:

/DD/LOG                    uucp log files

/DD/SYS/UUCP               uucp systems files directory

/DD/USR/SPOOL              spooled work directory

/DD/USR/SPOOL/MAIL         mailbox directory

/DD/USR/SPOOL/NEWS         Usenet news directory

/DD/USR/SPOOL/UUCP         uucp spool directory

/DD/USR/SPOOL/UUCPPUBLIC   uucp public spool directory

/DD/USR/SPOOL/UUCP/SAND    spooled work to/from "sand"

/DD/HOME                   user login directories

On some systems, the user login directories are in

/DD/USER. If you already have user home directory on

your system, you do not need the create a different

one.  There does not seem to be a standard on OS-9. :-(

==NOTE== The directory SAND is only an example.  Each

UUCP system you talk to needs its own directory in

/DD/USR/SPOOL/UUCP.

RICK    On Rick's original UUCP, the following

directories were used:

/DD/SYS/UUCP                uucp system files directory

/DD/SPOOL                   spooled work directory

/DD/SPOOL/MAIL              mailbox directory

/DD/SPOOL/NEWS              Usenet news directory

/DD/SPOOL/UUCP              uucp spool directory

/DD/SPOOL/UUCPPUBLIC        uucp public spool directory

/DD/SPOOL/UUCP/SAND         spooled work to/from "sand"

/DD/USER                    user login directories

You can keep this directory structure and simply

create the /DD/LOG

UUCPbb MANUAL                                                    8

Step 6   Set the directory permissions as follows:

/DD/LOG                       owner read, write, execute;

public write

/DD/HOME                      owner & public read, write,

execute

/DD/SYS/UUCP                  owner read, write, execute

/DD/USR/SPOOL                 owner read, write, execute;

public read, write

/DD/USR/SPOOL/MAIL            owner & public read, write,

execute

/DD/USR/SPOOL/NEWS            owner read, write, execute;

public read, write

/DD/USR/SPOOL/UUCP            owner read, write, execute

/DD/USR/SPOOL/UUCPPUBLIC      owner & public read, write,

execute

/DD/USR/SPOOL/UUCP/SAND       owner read, write, execute

Step 7   Time to copy some files.

RICK  BEFORE copying the files save copies of all your

original files in /DD/SYS/UUCP to a temporary

directory or on to a floppy.

Change to the directory /DOCS/SAMPLE in your current

data directory.  Copy all the files in this directory

to /DD/SYS/UUCP.

Step 8   Change to the directory /DD/SYS/UUCP

You are now ready to edit the various system files.  This may

sound more complicated than it really is.  Just take your

time.  Be sure to read the specified docs files before making

changes.

3.2 - Parameters File

Start up your text editor on the file: Parameters.  Refer to

the file parameters.doc while editing this file.

3.2.1 - Node Name

You'll need to pick out a name for your UUCP site.  You can't

pick out just any name; it has to be different than any of the

other some 100,000 sites out there already.  Needless to say,

this means that all the good ones are taken.  ;-)

This name is fairly important.  It must be from 1 to 8 (but no

more than 8) alphanumeric characters.  Punctuation or embedded

spaces are not allowed.  Many system administrators pick a name

which has special meaning to them.  For example, my node name

is "kc2wz" which is my Amateur Radio (ham) callsign.  Upper and

lower case are treated the same.  Lower case is normally used

when referring to a node.

9    Node Name                                           OS-9 UUCP

It's fairly easy for a UUCP administrator at an existing site

to check on possible name conflicts and to find all the UUCP

nodes close to you.  If you don't happen to know one, try

sending mail to me: bob@kc2wz.bubble.org.  Tell me what city

and state you live in.  I'll try to get a list of nodes near

you, with phone numbers and names of people at each site to

contact.

3.2.2 - Sitename

The sitename is not the same as your node name although it also

refers to your machine.  UUCPbb uses this parameter to identify

your fully qualified domain name.  In order to have a fully

qualified domain, you need a feed which is connected on the

Internet who willing to act a gateway to the Internet for you.

UUCP does NOT mean you are connected to the Internet or on the

Internet.  If you do not have a domain name, you use your

nodename with '.uucp' attached to the end for this parameter.

3.2.3 - Other parameters

Using the parameters.doc file a guide, you can finished editing

the Parameters file.  When you have a UUCP feed, you will need

to correctly fill in the uucphost, newshost, and inhost or

bithost parameters.

3.3 - Dialers File

The next file to edit is the Dialers file.  Dialers contains

info on all your modems.  The last field in the Devices file

refers to an entry in Dialers.  The sample Dialers file

explains the format of the entry.  If you are simply going to

use a Hayes compatible modem without any of its special

features such as MNP-4, MNP-5, etc, the file can be left

unchanged for now.  The 'hayes' entry will suffice.  If you

wish to take advantages of your modem's additional features,

the sample file will give you an idea of how to create other

entries.

3.4 - Devices file

The next file to edit is the Devices files.  Devices contains

info on all your serial ports.  The third field in the Systems

file refers to an entry in Devices.  With this file you can set

up the various ports of your system to handle more than one

modem.  Or it gives you a way to use the same modem and port in

various configurations.  The entry 'Hayes' will suffice for

situations which do not take advantage of a modem's special

features, such MNP-4.

UUCPbb MANUAL                                                   10

3.5 - bin.list File

This is a simple file which contains a list a extentions

commonly used to indicate binary files.  Mailx and FileServ

check bin.list to see if a file needs to be uuencode before

sending it.  You can add to this file if you need to.

3.6 - Systems

The Systems file contains info on how to access the remote

site.  There must be an entry for each of the UUCP systems that

you call and/or calls you.  The sample file gives the details

for the entry format.

If the remote system has more than one phone number, you need

to create a separate entry for each number.  UUCICO will then

call each number in the order it occurs.

The sample Systems file explains the entry format.

3.7 - Miscellaneous Files

There are a few miscellaneous files which UUCPbb uses.  Not all

of them are required, but a few are.

3.7.1 - Environment Variables

UUCPbb makes use of environment variables.  The ones used are:

HOME, MAIL, EDITOR, SHELL, and TERM.

HOME is the directory containing the users' login

directories.

MAIL is the directory containing the users' mailbox

directories.

EDITOR is the name of the text editor to be used when

composing mail or news articles.

SHELL is the name of the user's shell.

TERM is the type of terminal the user is on.

The CoCo does not have environment variables, so we need to

emulate them.  The default system environment on the CoCo is

kept in the file: /DD/SYS/profile.  Each user on the CoCo has a

'profile' file in their home directory which contains the

user's environment.  See the doc file: profile.doc for

information on setting up these files.

OSK has true environment variables.  The system administrator

of the machine needs to set MAIL, EDITOR, SHELL, HOME, and TERM

for the each user.  You will need to refer to your OSK docs on

how to do this.

11    Other Files                                        OS-9 UUCP

3.7.2 - Other Files

UUCPbb makes use of other files such aliases and mailrc.  There

are separate doc files which describe the contents of this

files so it will not be repeated here.

On the CoCo, each individual's UUCP related files, except

'profile', are kept in the directory UUCP. This is subdirectory

in each user's login directory.

On an OSK machine, the individual's UUCP related files are kept

in each user's root login directory.

3.7.3 - Password File

Your password needs to be properly set up for users to be able

to login and read or send mail or news.  User ID 0 in the

password file is the "superuser" account.  You may name this

account after yourself, or you may name it "root" or "sysop".

Currently, UUCICO runs as the superuser so you need at least

this account in the password file.  You should create a

separate account on your system for use when reading/posting

email and news.  Leave the the superuser account for doing

program development and system maintenance.

If you plan on accepting incoming UUCP calls, you will need to

create an that remote systems log into.  In the UNIX world,

this account name is often "nuucp". It does not always have a

password since the only program this account should every run

is UUCICO

Also make sure there is a "postmaster" account, too.  Any

incoming mail that goes to a name that can't be found in the

password file goes to "postmaster". Also, "postmaster" is an

established standard name on most UNIX sites to which

administrative mail is sent.  In other words, if someone at

another site wants to send mail to someone in charge at your

site, not just Joe User, and they don't want to spend the time

figuring out just who IS in charge over there, they send to

"postmaster". This user is defined in the

/DD/SYS/UUCP/Parameters file by the parameter 'errorsto'. It is

not a good idea to try to use an alias to redirect mail to

another user instead of making a separate account.

Another account is "daemon". This user is for future expansion

of UUCPbb.  It is not a regular user and receives no mail.  It

will be used it future updates to run certain tasks.

You can use the utility ADDUSER to easily create new accounts.

A sample /dd/sys/password file looks something like this:

root,rootpass,0,128,/dd/cmds,/dd,ex shell

nuucp,uucppass,1,190,/dd/cmds,/dd/usr/spool/uucppublic,ex uucico -r

postmaster,postpass,2,128,/dd/cmds,/dd/user/postmaster,ex shell

rickadams,rickpass,3,128,/dd/cmds,/dd/user/rickadams,ex shell

user1,rickpass,4,128,/dd/cmds,/dd/user/user1,ex shell

user2,rickpass,5,128,/dd/cmds,/dd/user/user1,ex shell

UUCPbb MANUAL                                                   12

3.8 - Installing Mail

3.8.1 - New Installations

If you are installing UUCP for the first time, you may need to

manually create the user mailboxes.  The main mailbox directory

is defined by the parameter 'maildir' in the /DD/SYS/Parameters

file.  Inside this directory are subdirectories.  The names of

these subdirectories are the same as the login names of each

user.  These are the user's mailboxes.  Each message is kept as

a separate file in the user's mailbox directory.

If you already have users installed on your system, you merely

need to create a mailbox directory for each user.  It is

important that the directory is owned by user it is named for.

That is, the user ID in the password file and the owner ID as

give by 'dir e' ('dir -e' for OSK) must be the same.  If they

are not, the user will not be able to read their mail.  On the

CoCo you can use the CHOWN utility included with UUCPbb to set

the ownership of the mailbox.

If you do not have users installed on your system, you need to

run the ADDUSER utility.  ADDUSER creates the necessary

directories such as the mailbox and home directories.

If you choose to use Jeff Shepler's mailer, Palm, (available on

chestnut.cs.wisc.edu), follow Jeff's instructions for compiling

and installing it.

3.8.2 - Upgrading Rick's mail

RICK   -If you are upgrading from Rick's original UUCP on the

CoCo, you have two choices.  If you are running Rick's original

mail program, you will need to run the utility CNVRTMAIL. See

the doc file CNVRTMAIL.MAN for details on how to use the

utility.  The mail format will be changed from the old form to

the new one.

3.8.3 - Upgrading Palm

RICK   -If you are upgrading from Rick's original UUCP on the

CoCo and are using Jeff's mailer, Palm, you need to use Jeff's

conversion utility to change the Palm for use with UUCPbb. The

utility and instructions are in the file CONVERTPALM.LZH which

is included with UUCPbb.

3.9 - CRON/MCRON

Although UUCP can be run manually, it really is meant to be run

by a task scheduler.  UUCPbb comes with Boisy Pitre's task

scheduler MCRON. You can also use CRON which is available on

chestnut.cs.wisc.edu, Delphi, and Compuserv as well a number of

landline BBSs. Some OS-9 BBS software, especially if it is

capable of handling Fidonet, often has some sort of event

scheduler as part of the software.  Any of these can be used to

run the various UUCP tasks.

13    CRON/MCRON                                         OS-9 UUCP

For example, here part of my crontab file:

#     Daily

1 0 * * * /dd/etc/script/dailycleanup&

# expire Usenet articles

30 4 * * * expire&

#     UUCP polls

40 0,3,6,9,12,15,18,21 * * * uucico fdurt1 -t3&

15 3 * * * uucico hvymetal -t3&

Some explanation is in order.  dailycleanup is a shell script

which runs daily housekeeping tasks, such as running UUCLEAN.

This occurs at one minute after midnight every day.  EXPIRE

runs at 4:30 am every day to delete old Usenet news articles.

UUCICO calls the remote site 'fdurt1' at 40 minutes after the

hour every three hours starting at midnight of every day.

UUCICO also calls the remote site 'hvymetal' every day at 3:15

in the morning.  The call to each remote is tried three times

before giving up for that session.

If any mail was received during the call, UUCICO will fork

UUXQT to process the mail.  This works fine on the

CoCo--usually. Some BBSs have the modem kill switch set to ON.

The result is that when UUXQT is chained to by UUCICO, it gets

killed off.  As an attempt to prevent this, you can try using

the -o option of UUCICO. This should cause UUXQT to be started

as an orphan.  Without a parent, the modem kill switch should

not effect it.  This option has not been thoroughly tested yet,

so bug reports and/or fixes are welcome.

The other alternative is to put a separate entry in the crontab

file to run UUXQT as certain times after a call to your feed is

through.  If you run UUXQT with the system name of 'ALL', all

the remotes listed in the Systems file will be check for newly

arrived work.

3.10 - Chat Scripts

Take a look at this example Systems file:

sandstorm Any sacia 2400 555-1234 ogin: uucp word: glorp

The sixth and subsequent field(s) are called the "chat script"

for that system.  This is a number of "expect/send" pairs,

giving what to expect the other system to say when you try to

log in, and what to say back in response.

So in the above example, we would wait till we saw the string

"ogin:" (actually, "login:"; chat scripts are usually

abbreviated this way), then respond with "uucp". The remote

system would then say "word:" (actually, "Password:"), and

we'll respond with "glorp" (the password).  This gives step by

step directions for logging into the remote system that are

used by uucico when it calls it up.  There is a similar chat

script in the Dialers file entry for a specific modem, starting

with the second field.

UUCPbb MANUAL                                                   14

There are a number of special sequences you can put in chat

scripts:

""          expect (or send) nothing

\b          a backspace character (hex 08)

\d          a 1-second delay

\s          a space (hex 20)

\r          carriage return (hex 0D)

\n          a linefeed character (hex 0A)

\N          a NUL character (hex 00)

\T          the phone number given in the Systems file

\\          a backslash

\t          a TAB character (hex 09)

\NNN        ASCII character NNN in octal

\^?         send a control character, for example:

\^A sends Ctrl-A

\c          suppress carriage return at end of

send string

A carriage return is automatically sent after each "send"

string unless the "c" sequence is used at the end of the

string.

3.11 - Preventing Lockups

Included in UUCPbb is a patch written by Boisy Pitre for the

CoCo. This file is call SCF14.AR. Applying this patch to SCF

allows you to make your serial ports non-shareable.  Why would

you want to do this?  Well, let's say you are calling a BBS and

CRON decides it is time for UUCICO to make a call--on the same

port.  When your terminal program and UUCICO try to use the

same port, things will lock up real tight!  With the

non-shareable patch installed, UUCICO will be able to detect

that the port is already in use and not interfer.

Right now, I'm an not sure how to accomplish the same thing

under OSK. If someone has the proper procedure, please let me

know.  In the meantime, OSK users will have to be aware that

there may be a conflict in a situation similar to the above one

happens.

15    Testing                                            OS-9 UUCP

4 - Testing

Now that you have reached this point, you should be ready to

see if it all works.

4.1 - Finding a Feed

In order to test UUCICO, you will need to find an existing UUCP

system willing to provide you with a UUCP mail feed.  Tell them

you'd like to poll their system a couple times a day to send

and pick up your mail.  The important thing to get across is

that you'll be dialing THEM, on your dime, you'll be only

calling them a few times per day, and the volume will be fairly

low, since you're a one-person "leaf" site.  (A "leaf" site is

one with only one UUCP connection.) In other words, you're only

picking up mail for one person, and not providing mail service

to lots of other people downstream from you.

They'll also be concerned that you call in regularly, so that

your UUCP traffic doesn't queue up endlessly on their system

waiting for you to call in.  If you sort of forget about

calling in for a few weeks, and a few hundred messages spool

for you on the remote system, you will most probably get a

concerned and/or annoyed phone call.  :-) Or they mail yank

your feed altogether.  Remember, you are requesting to use

their system, not abuse it.

Also, make sure they understand you are NOT a business, NOT a

BBS (unless, of course, you ARE one :-) ), you are an

individual doing this for hobby purposes.  If they think you're

a business, they'll likely tell you to go somewhere where you

can hook up to a UUCP site for a fee, since you ought to be

able to pay for it.  If they think you're a BBS, they'll be

worried about how much traffic they're going to be saddled

with, at their expense.

(On the other hand, some site administrators are sympathetic

towards BBS operators, and will be very helpful.  It depends on

the person.)

How do you find such a site?  You can look in the UUCP map data

for your state for all the sites near you, then call the

contact phone numbers provided in the map data trying to find

someone agreeable to your request.  They may say no, they may

give you a mail feed for free, or they may offer to provide it

for a fee.  If they agree to give you a feed for free, you're

getting quite a deal, and you definately should put that person

on your Christmas card list.  :-)

This is a Catch-22, of course.  You can't set up a UUCP node

unless you can find a site to connect to, but you can't find a

site to connect to unless you have access to UUCP. The solution

is to somehow contact someone who already has access to a UUCP

site and who is also fairly knowledgeable about how it all

works.  If you can find such a person (asking around at most

universities usually yields paydirt), so much the better.  If

not, well, send me email (bob@kc2wz.bubble.org) as I'll see it

I can help.

UUCPbb MANUAL                                                   16

UUCP administrators tend to be technically minded middle-level

people, many of whom have a soft spot in their heart for eager

young "hackers" (in the original sense of the world), and an

affectionate and almost paternal love of their system, to the

point that you'll be made to feel like you've just asked to

take out one of their daughters.  And, in a way, you have. :-)

Be cheerful, respectful and polite, and see what happens.

The UUCP administrators at the site you connect to may ask you

to only call during certain specific hours, to not tie up

his/her modems during normal business hours.  That's a common

restriction; agree to it if asked, and don't even think about

fudging on it.  UUCP keeps log files of all the traffic to and

from your site, so he/she can check up on you, and probably

will.  Blow it, and you may find your email feed yanked in a

hot minute, and you'll be out looking for another site all over

again.

If you find a site willing to provide a mail feed, you'll be

given:

One or more phone numbers to a modem on their system

The UUCP node name of their system

An account and password for UUCP access

Hours in which you may access the system

The UUCP coordinator may want to know:

Your voice phone number

Your modem phone number

The name of your UUCP node

If they ask you "what is your modem number, and when can I call

you", tell them that you'll be calling THEM, and that they

should put "Never" as the time they can call you.  This is

usually more handy than having them call you.  Or, if you want

your machine to be on a lot, tell them the hours you can accept

UUCP related calls from their machine.  There is a way you can

set up your system to accept UUCP calls in "slave" mode, so

that your UUCP neighbor can connect to your machine in the

background.  This will be covered shortly.

4.2 - Running as Master

When a system calls out, it is running in the Master mode.  In

order to do this you will need to have your modem set up to

return the result codes in English. UUCICO looks for the

"CONNECT" string to let it know the remote modem has answered.

You should also have your modem set up to drop the connection

if the remote modem hangs up.  The entries for 'hayes' in the

Device and Dialers files should work with most situations.

Sometimes, however, it is necessary to play with the modem

settings to get it to work properly.  The settings in Devices

and Dialers for 'zoom' are for a Zoom V.42bis modem.  These

settings have worked very well for more than a year on my

system.

17    Running as Master                                  OS-9 UUCP

Once you have installed the system and setup the various files

and have at least one real UUCP site in the Systems file, you

are ready to test your installation.

Be sure that you do not have TSMon or any other process running

on the port your modem is connected to.  Begin by typing:

uucico sitename

If everything is setup correctly, UUCICO should dial the remote

site, connect, login and perhaps even transfer some mail.  Once

it finished type this from the OS-9 prompt:

OS9: uulog

You should get something like this on your screen:

uucp fdurt1 (Mar 28-15:30:29) Calling system fdurt1 on port /t2

uucp fdurt1 (Mar 28-15:30:55) Login successful

uucp fdurt1 (Mar 28-15:30:56) Handshake successful (protocol 'g')

daemon fdurt1 (Mar 28-15:30:57) Request to send 'D.fdurt1b10aM'

daemon fdurt1 (Mar 28-15:31:45) Request to send 'X.fdurt1X10aK'

uucp fdurt1 (Mar 28-15:31:55) Call complete (76 seconds)

uuxqt fdurt1 (Mar 28-15:32:05) rmail D.fdurt1b10aM "kc2wz!bob"

If you see anything else or a FAILED in the "Call complete"

line, something is wrong and you will have to correct it.  The

error message are pretty much self-explanatory.

If you failed to login, you probably have something wrong in

the chat script.

Getting the chat script right is sometimes the hard part.  Try

calling the remote while using a terminal program.  Capture the

login sequence without any sort of filtering.  Note the

sequence you have to follow in order to properly login.  Once

you can login yourself, just duplicate the sequence in your

chat script.

Make sure the system administrator of your feed (and you) have

your login name and password correct.  A difference of only one

letter will prevent you from logging in.

If you logged successfully, but UUCICO fails afterward, you can

run UUCICO with a higher debug level to narrow down the

problem.

If everything went well, congratulations!!  Welcome to the UUCP

world.  :-)

UUCPbb MANUAL                                                   18

4.3 - Running As Slave

When a system is set up to take incoming UUCP calls, it is in

the Slave mode.

To run your machine in slave mode, create a line like this in

your /DD/SYS/password file:

nuucp,uucppass,2,190,/dd/cmds,/dd/usr/spool/uucppublic,ex uucico -r

The user number in this example is "2", but this is not

important.  Note the higher priority of 190, and the "uucico

-r" command.  "-r" specifies slave mode.  Use your own password

instead of "uucppass", of course.

Then you need to set your system and modem up for slave mode.

Create a shellscript similar to this:

echo ate0q1s0=1 >/t2

tsmon /t2&

echo done

Any UUCP neighbor that wants to call you while you're in slave

mode will need a "chat script" for your system, to know how to

log in.  Here is a sample one.  This example assumes my modem's

phone number is 555-1234, my uucp account is "nuucp", with a

password of "itsme":

kc2wz Any hayes 2400 5551234 "" r ogin: nuucp word: itsme

This line, or one very much like it, will go into the "Systems"

file on your UUCP neighbor.  At this point that system will

know how to call your system and log in using slave mode,

whenever it might feel the need to do so.  This assumes that

your system is ready to accept the call.

If you don't plan on accepting calls in slave mode at all, but

just want to call them, tell them to put "Never" in that

field.

If your systems needs to call out, it will first have to kill

TSMon on the port, other UUCICO will not be able to call out.

19    Setting Up News                                    OS-9 UUCP

5 - Setting Up News

You need to create the following files:

1.   /DD/SYS/UUCP/active

This file will contain a list of all the Usenet newsgroups

you want to receive, one per line.  When setting up your

system, format each line as in the following example:

rec.arts.anime 0 0

comp.sys.m6809 0 0

sci.astro 0 0

comp.os.os9 0 0

(Each line consists of the newsgroup name followed by the

minimum and maximum article number in the system, which

starts out at 0 and 0 for a new newsgroup.)

2.   /DD/SYS/UUCP/distributions

This file contains a list of all the Usenet distributions

for your site, one per line.

Usenet "distributions" control how widely your posted news

article will be sent.  Some distributions are, for

example:

world       anywhere in the world

na          north america

usa         the united states

You will have other distributions in your area, as well.

Here are the distributions for my area:

nj          New Jersey

ny          New York

ne          Northeast

Typically there will be a statewide distribution for your

state named after its two-letter code, i.e.  "tn" for

Tennessee, etc.

So figure out all your local distributions, then add them

to the world, na and usa distributions, and put them in

the distributions file, one per line, like this:

world

na

usa

nj

ny

UUCPbb MANUAL                                                   20

5.1 - Turning On Your News Feed

Once you figure out which newsgroups you want, you have to tell

the system administrator of your UUCP neighbor what you want to

get.  Tell him/her, for example, "I'd like a batched,

compressed newsfeed of comp.os.os9, comp.sys.m6809, and

ba.test."  (This is an example ONLY; check to see what

newsgroups would be best for your tastes, and for your area.)

This implementation can handle batched or unbatched,

uncompressed or uncompressed news, but batched compressed news

is most efficient.  The OS-9/6809 can only handle 12-bit

compression, which is usually the default.  OSK systems can

handle 16-bit compression.  You must have the utility COMPRESS

in your execution directory.  OS-9 COMPRESS is available on

Delphi, Compuserve and chestnut.cs.wisc.edu.  Do not use the

16-bit version of COMPRESS for OS-9/6809, it is WAY too slow

for news.

You won't receive any news, or be able to post any news, until

your UUCP neighbor adds a line to their /usr/lib/news/sys file

that specifies that your system is to receive the specified

newsfeed.  Also, if your neighbor doesn't receive one or more

of the newsgroups you request, you won't be getting any

articles for it, either, and the administrator will probably

point that out.

The administrator will add your line to his/her sys file in

response to your request for a news feed.

You'll eventually start receiving your news articles, usually

in one big chunk once per night.  If you run uucico with debug

output turned on, you'll see "rnews" getting executed at that

point, in addition to the usual "rmail."

Make sure all your files are in readiness on your site before

you request your newsfeed turned on; you'll have a heck of a

time once the articles start flowing in, otherwise.

6 - UUCP?  What is it?  How Does It Work?

6.1 - What UUCP is NOT

UUCP is NOT a BBS program.  UUCP is NOT a terminal program.

Running UUCP does NOT mean you are connected to the Internet.

Although some sites on the Internet provide a way to access

their services via email, this does not mean you are on the

Internet. Connecting to the Internet is a bit more complicated

the running UUCP.

21    What UUPC is                                       OS-9 UUCP

6.2 - What UUCP is

UUCP stands for "Unix-to-Unix Copy", and was an early file

transfer protocol supplied with most versions of the UNIX

operating system.  It has since been adapted to run on numerous

other machines as well, including IBM PC's, Macintoshes,

Amigas, and so on.  UUCP is more properly a suite (collection)

of programs which work together to transfer mail, news and

files.

Each computer running UUCP is known as a UUCP "site", or

"node". There could be one or may users on a particular node.

Each UUCP site has one or more "UUCP neighbors" with which

files, email, and possibly Usenet news articles are exchanged:

* Files can be transferred from one system to another via

the UUCP command.

* Email can be sent or received.  It travels from system

to system, one "hop" at a time, until it reaches its

intended destination.

* Usenet is a worldwide network for the exchange of

messages, sort of like a very large, diversified,

sprawling BBS. Usenet "newsgroups" exist that are

devoted to just about any conceivable topic.  If one of

your UUCP neighbors receives a Usenet newsgroup on a

topic you're interested in, you can arrange to have it

sent to your site too, and can post messages to that

newsgroup that are sent out for worldwide distribution,

as well.

Some people may recognize this as being somewhat similar to the

way that the Fidonet network works.

Each UUCP node has a unique name.  Email is sent to users at

another site by addressing the mail to the "path" to that site,

which consists of the sequence of sites between your site and

the ultimate destination site, followed by the user account

name on the destination, separated by exclamation marks.

For example, in the following hypothetical example, the nodes

"coco3", "sandstorm", "gigsys", "papabear", "homebase",

"garage3", and "tatooine" are interconnected in the following

manner:

coco3----------sandstorm-------------gigsys----------papabear

\              /  \                 \              /

\            /    \                 \            /

\          /      \                 \          /

\        /        \                 \        /

\      /          \                 \      /

homebase       garage3              tatooine

UUCPbb MANUAL                                                   22

So, "coco3" has two UUCP neighbors, "sandstorm" and "homebase".

Site "gigsys" has three UUCP neighbors: "sandstorm",

"papabear", and "tatooine". The site "garage3" is what is known

as a "leaf" site: it is connected to only one UUCP neighbor,

"sandstorm". It is a "leaf" on the interconnection "tree" in

that it only generates and receives email (and possibly Usenet

news) for itself, and doesn't pass any traffic through for

anyone else.

So if someone at site "coco3" wanted to send email to someone

with the user account "susan" at site "garage3", they'd send it

to:

sandstorm!garage3!susan

If someone at site "homebase" wanted to send mail to user

"fred" at site "tatooine", they'd send it to:

sandstorm!gigsys!tatooine!fred

Suppose someone at site "papabear" wanted to send email to user

"sally" who was also at site "papabear"?  In that case, it

becomes rather simple.  They'd send the email to:

sally

Email sent to someone on a remote system is queued up to be

sent.  On a regular basis, you call up one or more of your UUCP

neighbors, at which time the two nodes connect and exchange any

files that are waiting to be sent or received by one or the

other.  After this is done, the two computers disconnect, and

each then processes any incoming files that require further

work.  Email may need to be distributed to a user on that

system, or queued up for transmission to yet another UUCP

neighbor, for example.  Or a Usenet news article may need to be

posted on your machine.

This is a hypothetical example.  Multiply that diagram a

hundredfold, and include tens of thousands of UUCP sites around

the globe, and you'll have a more accurate picture of exactly

what it is you're tapping into.

You and any other users on your machine will mostly use the

MAILX, UUCP, READNEWS and POSTNEWS commands.

As a Sysop on your system, you will run the "uucico" command

once or twice a day to connect with your neighbors and get

everyone up to date.  Also, when your disk starts getting full,

you'll want to run EXPIRE to delete Usenet news articles older

than a specified number of days old.

UUXQT, RMAIL and RNEWS are usually run automatically by

UUCICO.  UUXQT is run after "uucico" has transferred all

files to and from your site that have been queued up for

transport, and you have disconnected from your UUCP neighbor.

It then runs RMAIL and RNEWS, as appropriate, to post any

email or news articles that may have just come across.

23    How Do You Get There From Here?                    OS-9 UUCP

6.3 - How Do You Get There From Here?

To send email to someone, you need to know the name of their

node, and the names of all the nodes between yours and

his/hers.  If your friend Sally had an account "sallyd" on

system "burp", and "burp" was connected to system "why", and

system "why" was connected to "sandstorm", you'd do this:

mailx "sandstorm!why!burp!sallyd"

Finding a path between your system and another is an arcane

art.  :-)

The quotes are necessary to prevent the OS-9/6809 shell from

interpreting the exclamation points as "pipe" characters.

A single dot character at the left margin will terminate the

mail entry.  Mail then queues the mail up to be sent to

"sandstorm" the next time you call them with uucico.

7 - UUCP Command

The uucp command can be used to initiate file transfers to and

from the remote system.  The remote system usually won't let

you put a file just anywhere...  usually there is a

/usr/spool/uucppublic directory to which anyone is allowed to

write files, though.  So to transfer your startup file to your

immediate UUCP neighbor "sandstorm", for example:

uucp /dd/startup "sandstorm!/usr/spool/uucppublic/startup"

and to read it back:

uucp "sandstorm!/usr/spool/uucppublic/startup" /dd/startup2

This would just queue up a job to do the transfer; uucico would

do the actual work.  This means that people who know what

they're doing can send you files (and read your files)

remotely, so you might want to be security conscious and NOT

run uucico from your root account (user 0).  Also, make sure

the permissions on your files are set such that whatever

account you run uucico under can ONLY do things on your system

that you feel like permitting.

The directory /DD/USR/SPOOL/UUCPPUBLIC is your version of

standard /usr/spool/uucppublic. Make sure anyone can write in

that directory, so that anyone wanting to transfer a file to

your system will be sure to have a place to put it.

If you know enough about the way the network's connected, and

the various machines involved are configured in a way that

allows it, you can transfer files between machines that aren't

immediate neighbors, like this:

uucp a_file "sandstorm!gigsys!tatooine!/usr/spool/uucppublic/file"

UUCPbb MANUAL                                                   24

The above command would cause the file "file" to be transferred

from your hard disk to the /usr/spool/uucppublic directory on

system "tatooine", travelling there by ways of intermediate

systems "sandstorm" and "gigsys".

8 - File Server

Rick Adams has written a mini-listserver program which is

included in UUCPbb.  At the present time the server is somewhat

simple.  It will allow users to request files you have placed

in a special directory.  This directory is defined in the

/DD/SYS/UUCP/Parameters file.

Read the file FILESERV.MAN for full instructions on using

setting up and using the server.

While the server is simple now, it will not be for long.  ;-)

9 - Final Word

I would like to see UUCPbb develop into an OS-9 Community

Project. Towards that end, I have released it under the GNU

General Public License. You are free to give away (but NOT

sell) and modify the source or even use the source provided you

comply the the GNU license.

I will be maintaining UUCPbb, so if you have bugs,

improvements, comments, critisms or even nice things to say

:-), please pass them know.  I don't have the last word on how

to do everything in C. If someone can come up with a better

(and PORTABLE!) way of doing what I did, let me know.

In order to avoid requiring everyone to constantly download a

huge file for minor updates or wait a long time between

revisions, a UNIX-like diff utility is in the works.  (It will

run both on the CoCo and OSK systems.) With this utility

changes will be easier to pass along.  Any DIFF files will

ALWAYS be referenced against the current official version of

UUCPbb. This will ensure that everyone is working with the same

sources.  The original site which updates, patches, additions

will be on the OS-9 archive site on the Internet. Currently,

this site is chestnut.cs.wisc.edu.  I am hoping a separat

directory will be created for UUCPbb related files, so y'all

won't have to search all around the directories on chestnut.

From chestnut, files will migrate to Delphi, Compuserv and

other BBSs.

If you want to help with this project, contact Jeff Shepler at

sysop@miliways.aldhfn.org.

There are other things I would like to do with UUCPbb to make

it better.  Check the TODO list.  If you see something you want

to work on OR you have, like me, been quietly hacking on Rick's

code and come up with something you want to contribute, drop me

email.  Before you start working on something check with me

first.  Someone else may be working on same task.  Rather

25    Introduction                                       OS-9 UUCP

pointless to two people duplicating each others work when you

could working together.

Obviously, there are some things that you can do under OSK that

you can't do due to limitations of the CoCo and OS-9/6809.

Where possible please try not to make none portable changes.

If you do, please use the following #ifdef/#endif to allow

selectively compiling the code:

#ifdef OS9    Code specific to OS-9/6809

#ifdef _OSK   Code specific to OS-9/68K or OS-9000

#ifdef _OS9K  Code specific to only OS-9000

So put on your thinking caps!  And have fun!

Bob Billson <bob@kc2wz.bubble.org> 1994 April 2